Instructions for TLSO (Clam shell) with Aspen Attachment 
The Clam Shell Orthosis is designed to restrict all motion of the mid and lower (low thoracic and lumbar) spine. When combined with the attached Aspen collar, it also restricts motion in the mid-thoracic spine.  

[image: image1.png]



How to applv: (in flat lying) ***Patient should wear an Aspen Vista collar while lying in bed.
Donning the TLSO section:
1. Ideally the TLSO part of the brace should be worn on top of a T-shirt, nightshirt, or long undershirt to allow for ventilation. 
2. Log roll the patient to their preferred side. 
3. Put on back section of the brace first, by sliding edge of brace and velcro straps under the patient's side. The bottom of the orthosis should cover 2/3 of the bum.  The bottom strap should lie below the level of the hip.  
[image: image2.png]



4. Roll patient flat onto their back, making sure that the shoulders and hips are square. At this point, the TLSO back may need to be pulled so that the it is lying centered on the back. Feel the sides to make sure that the orthosis comes up equally on either side of the body.  Never lift the hips AT ANY TIME.
5. [image: image3.png]


Next place the front section of the TLSO on the patient. This section should be placed low enough so that the belly is covered as much as possible. This may mean that the chest section is lower than normal, but it will move up once the brace is tightened.  Straps should be done up from the bottom up, loosely at first, and then pulled tight.  The bottom one should be very tight, the middle tight, and the top one snug. The wearer should be able to breathe normally, but NOT be able to take a deep breath through the belly.
[image: image4.png]





Turn the page over
[image: image5.png]


[image: image6.png]


Donning the Attached Aspen Collar:
1. Remove the standard Aspen Vista Collar.  Make sure that the patient does not turn their head.

2. Place the Aspen Collar with the stiff extension onto the front of the neck, and pat the Velcro onto the front of the TLSO front.   
How it should look: 


TLSO Section: When wearing the brace in sitting, it should sit on top of the thighs, and the 
back of the brace should be about 1" above the seat of a firm chair. In standing, the back 
section should end at the height of the curve in the buttocks, and the midline should be within 
1" of either side of the spine. If it is not there, it should be reapplied. 


There needs to be a gap between the front and the back section. If these sections touch on both 
sides, the brace needs to be adjusted. Please call AMPOS to arrange to have this adjustment 

Aspen Section:  The person should be looking forward when sitting without lifting or lowering their head.  You can sometimes lift the Velcro section and set it so that the patient is looking forward.  If this does not work, the dial may need to be adjusted.
How to remove:  Lie down flat on the back.  Remove attached Aspen front first, and re-apply the standard Aspen front.  After the neck brace is re-attached, you can undo the Velcro straps on the TLSO section.  Remove front section first.  Log roll patient to most comfortable side.  Slide out back section. 

When to wear brace:  The brace should be worn whenever the patient is sitting up or standing. It can be off when they lie down. It should be applied and taken off in a lying position. 

Shower/Bathing:   The brace can be worn in the water. After a shower or bath, remove the brace, towel dry and let air dry for a short time (15- 30 minutes). We usually recommend washing at night, so the brace can dry completely. 

Care & Cleaning:   We recommend washing the TLSO section at least once a week, and the Aspen Vista pads should be changed and washed daily. Use soap and water and a washcloth or sponge. Rinse the soap residue off completely. After cleaning, towel dry and let stand for a short time (15- 30 minutes).  The Aspen pads may take longer to dry. Lay them on a towel to dry. 

If you have any problems, questions, or concerns regarding the brace please give AMPOS Orthopaedics a call at (613) 745-3173 or 1-800-216-6777.   To see an instructional video on how to don the TLSO section, go to: https://www.youtube.com/watch?v=fydK8Bf_oSw  If that link does not work, go to www.youtube.com and use the search engine to look for TLSO and AMPOS.
